Frames – v2

Topology 1

- Based on the information provided on the frame advise what information will properly complete the frame below:

| MAC ADDRESS | MAC ADDRESS | IP ADDRESS | IP ADDRESS | PORT | PORT | DATA |
| --- | --- | --- | --- | --- | --- | --- |
| 000B.BE5B.3CE8 | 0030.F2DE.7706 | - | - | 51029 | 23 |  |

  - Source IP 172.29.100.100 & Destination IP 172.29.100.10

  - Destination IP 172.29.100.8 & Source IP 172.29.100.100

  - Source IP 172.29.100.10 & Destination IP 172.29.100.100

  - Destination IP 172.29.100.100 & Source IP 172.29.100.10

- A frame going from pc0 going to laptop2 was captured:

| MAC ADDRESS | MAC ADDRESS | IP ADDRESS | IP ADDRESS | PORT | PORT | DATA |
| --- | --- | --- | --- | --- | --- | --- |
| - | - | 172.30.54.10 | 172.29.100.10 | 51029 | 20 |  |

Please select the layer 2 addresses that will properly complete the frame.

- Destination MAC 0001.c743.7402 & Source MAC 000a.f39e.3b01

- Source MAC 0001.c743.7402 & Destination MAC 000a.f39e.3b01

- Destination MAC 000a.f39e.3b01 & Source MAC 0001.c743.7402

- Source MAC 000a.f39e.3b01 & Destination MAC 0001.c743.7402

- A frame captured contained the following information:

| MAC ADDRESS | MAC ADDRESS | IP ADDRESS | IP ADDRESS | PORT | PORT | DATA |
| --- | --- | --- | --- | --- | --- | --- |
| 000a.f39e.3b02 | 0060.47C7.6C6D | - | - | 49512 | 22 |  |

Based on this information, which Layer 3 addressing option will properly complete the frame?

  - Source IP 172.29.100.100 & Destination IP 172.30.100.200

  - Source IP 172.30.100.100 & Destination IP 172.29.100.254

  - Source IP 172.30.100.100 & Destination IP 172.30.54.10

  - Source IP 172.30.100.200 & Destination IP 172.29.100.100

Topology 2

- Based on the information provided on the frame advise what information will properly complete the frame below:

| MAC ADDRESS | MAC ADDRESS | IP ADDRESS | IP ADDRESS | PORT | PORT | DATA |
| --- | --- | --- | --- | --- | --- | --- |
| 00D0.589C.1903 | 00D0.9764.6801 | - | - | 61000 | 80 |  |

Which Layer 3 addressing option will properly complete the frame

  - Source IP 172.26.27.10 & Destination IP 10.15.26.250

  - Source IP 172.26.27.10 & Destination IP 10.15.26.200

  - Source IP 172.27.26.10 & Destination IP 10.15.26.250

  - Source IP 10.15.26.250 & Destination IP 172.27.26.10

- A frame captured contained the following information:

| MAC ADDRESS | MAC ADDRESS | IP ADDRESS | IP ADDRESS | PORT | PORT | DATA |
| --- | --- | --- | --- | --- | --- | --- |
| 000C.CF9B.A303 | - | - | 192.168.10.12 | 50400 | 20 |  |

Based on this information which Layer 2/layer 3 addressing option will properly complete the frame

  - Destination MAC 00D0.5894.7AC3 & Source IP 192.168.10.151

  - Source MAC 0001.C75D.A0A3 & Source IP 192.168.10.151

  - Destination MAC 0001.C75D.A0A3 & Destination IP 192.168.10.151

  - Source MAC 00D0.5894.7AC3 & Source IP 192.168.10.151

- A from was captures going from ITN.com to Cashier-2

| MAC ADDRESS | MAC ADDRESS | IP ADDRESS | IP ADDRESS | PORT | PORT | DATA |
| --- | --- | --- | --- | --- | --- | --- |
| - | - | 10.15.26.250 | 172.27.26.11 | 53 | 53441 |  |

Please select the layer 2 addresses that will properly complete the frame

  - Source MAC 00D0.589C.1902 & Destination MAC 0090.0CBC.C802

  - Destination MAC 00E0.F7D8.CD83 & Source MAC 00D0.589C.1902

  - Source MAC 0004.9A14.1D9D & Destination MAC 0001.4285.4B4B

  - Destination MAC 0090.0CBC.C801 & Source MAC 00D0.589C.1902